# XYZ Testing Lab

# Random Vibration Test Report

SETUP NAME: ESS Random:  VLTS Program

RUN NAME: run002 - Serial Number MRW32-Q

SAVE NUMBER: 1

### STATUS INFORMATION

TEST EVENT TIME: Wednesday, August 03, 2016 at 04:00:53 PM

TEST STATUS: RUNNING

TEST MODE: AUTO

TOTAL TIME ELAPSED (HH:MM:SS): 00:45:28

AUTO TIME ELAPSED (HH:MM:SS): 00:44:48

TEST LEVEL: 0.0 dB

REFERENCE: 9.94 g rms

CONTROL: 10.01 g rms

### CONTROL PARAMETERS

CONTROL CHANNEL(S): 1

CONTROL TYPE: SINGLE

FREQUENCY RANGE: 2500 Hz

NUMBER OF PSD LINES: 1000

FREQUENCY RESOLUTION: 2.500 Hz

DOF: 100

AMPLITUDE DISTRIBUTION: GAUSSIAN

SIGMA DRIVE LIMITING: 3.00

### REFERENCE BREAK POINT TABLE

| BP # | Freq Hz | Level (g²/Hz) | Slope (dB/oct) | Alarm- (-dB) | Alarm+ (+dB) | Abort- (-dB) | Abort+ (+dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 20.00 | 0.002655845 |  | -3.00 | 3.00 | -6.00 | 6.00 |
| 2 | 135.00 | 0.06639612 |  | -3.00 | 3.00 | -6.00 | 6.00 |
| 3 | 350.00 | 0.06639612 |  | -3.00 | 3.00 | -6.00 | 6.00 |
| 4 | 475.00 | 0.04149758 |  | -3.00 | 3.00 | -6.00 | 6.00 |
| 5 | 650.00 | 0.07303574 |  | -3.00 | 3.00 | -6.00 | 6.00 |
| 6 | 1100.00 | 0.07303574 |  | -3.00 | 3.00 | -6.00 | 6.00 |
| 7 | 2000.00 | 0.01659903 |  | -3.00 | 3.00 | -6.00 | 6.00 |

Accel: 9.94 g rms, Est. Vel: 6.67 in/s pk, Est. Disp: 0.0385 in pp, Ref. Disp: 0.0336 in pp

### TEST STARTUP & SCHEDULE PARAMETERS

START LEVEL: -20.00 dB, (10.00% of full level)

AUTO RAMP RATE: MEDIUM

AUTO RANGE SETTLE PERIOD: FAST

AUTO MODE TIMER BEGINS AT LEVEL SCHEDULE STEP: ALL/ANY STEP(S)

TEST DURATION (HH:MM:SS): 01:00:00

### LEVEL SCHEDULE

| Step # | Level (-dB) | Level (%) | Duration (HH:MM:SS) |
| --- | --- | --- | --- |
| 1 | 0.00 | 100.00 | 01:00:00 |